# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 2

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 2- Debugging & Optimisation Task |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | P.AKSHAYA | Reg.No.: | 192525190 |
| Department: | AIML | Date: |  |

### ANNEXURE A

- A signed binary addition program is producing incorrect results when adding negative numbers using 2’s complement representation. Debug the logic to identify where overflow or sign handling fails, and optimize the implementation to correctly detect overflow conditions while maintaining efficient execution.

- A carry look-ahead adder circuit designed for fast addition is not achieving the expected speed improvement over ripple carry addition. Analyze the design to locate delays in generate and propagate logic, and propose optimizations to improve performance without increasing hardware complexity significantly.

- A multiplication module based on Booth’s algorithm is generating incorrect outputs for certain signed inputs. Debug the step-by-step execution of the algorithm to identify errors in bit shifting or decision rules, and optimize the implementation to reduce the number of required cycles.

- A floating-point arithmetic unit implementing IEEE 754 single precision is producing rounding errors and incorrect normalization. Identify the faults in exponent alignment, mantissa calculation, or rounding logic, and optimize the design to ensure accurate and efficient floating-point operations.

- A processors instruction execution pipeline (fetch, decode, execute) is experiencing delays due to improper instruction sequencing and hazards. Debug the pipeline stages to detect data or control hazards, and propose optimization techniques such as pipelining improvements or instruction reordering to enhance overall processor performance.

Code of Conduct:

IP.AKSHAYA.(192525190)	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

- Signed Binary Addition (2’s Complement)

Issue Identification:
Incorrect results with negative numbers usually occur when overflow is not detected correctly or when the sign bit is handled improperly.

Debugging:

- Verify that operands are represented in proper 2’s complement form.

- Check the carry into and carry out of the most significant bit (MSB).

- Overflow occurs when:

  - Two positive numbers produce a negative result.

  - Two negative numbers produce a positive result.

- Overflow detection formula:

Overflow = Carry into MSB ⊕ Carry out of MSB

Optimization:

- Use XOR-based overflow detection instead of additional comparison logic.

- Perform sign checking only on the MSB to minimize hardware overhead.

- Maintain fixed-width arithmetic for efficient execution.

2. Carry Look-Ahead Adder (CLA)

Issue Identification:
The expected speed improvement is lost when delays occur in generate (G) and propagate (P) signal computation.

Debugging:

- Verify:

  - Generate: G = A × B

  - Propagate: P = A ⊕ B

- Check if carry signals are computed sequentially instead of in parallel.

- Identify excessive gate levels in carry-generation paths.

Optimization:

- Compute generate and propagate signals in parallel.

- Use hierarchical carry look-ahead blocks for larger adders.

- Minimize logic depth in carry equations.

- Optimize gate fan-out to reduce propagation delay.

- Booth’s Algorithm Multiplication

Issue Identification:
Incorrect outputs are commonly caused by errors in arithmetic right shifting or incorrect application of Booth’s decision rules.

Debugging:

- Verify Booth encoding rules:

  - 01 → Add multiplicand.

  - 10 → Subtract multiplicand.

  - 00 or 11 → No operation.

- Check arithmetic right shift after every iteration.

- Ensure sign extension is preserved during shifts.

- Confirm correct initialization of accumulator, multiplier, and extra bit.

Optimization:

- Use modified Booth encoding to process multiple bits per cycle.

- Reduce the number of iterations by grouping bits.

- Implement efficient arithmetic shift hardware.

4. IEEE 754 Single-Precision Floating-Point Unit

Issue Identification:
Rounding errors and incorrect normalization typically result from faults in exponent alignment, mantissa operations, or rounding logic.

Debugging:

- Verify exponent comparison and alignment before addition/subtraction.

- Ensure mantissas are shifted correctly during alignment.

- Check normalization after arithmetic operations.

- Validate IEEE 754 rounding mode implementation (round-to-nearest-even).

Optimization:

- Use guard, round, and sticky bits for accurate rounding.

- Implement efficient leading-zero detection for normalization.

- Perform exponent adjustment and normalization concurrently where possible.

5. Instruction Execution Pipeline

Issue Identification:
Pipeline delays are caused by data hazards, control hazards, or improper instruction sequencing.

Debugging:

- Detect:

  - Data Hazards: Instruction depends on previous results.

  - Control Hazards: Branch instructions alter execution flow.

  - Structural Hazards: Multiple instructions compete for the same resource.

- Analyze stage interactions (Fetch, Decode, Execute) to locate stalls.

Optimization:

- Implement data forwarding (bypassing) to reduce stalls.

- Use hazard detection units for dependency management.

- Apply branch prediction to minimize control hazards.

- Reorder instructions where possible to avoid pipeline conflicts.

- Balance pipeline stages to improve throughput.

These corrections ensure accurate operation while maintaining efficient hardware performance.

Top of Form

Bottom of Form