# SIMATS ENGINEERING

Department of Computer Science & Engineering

## ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | P.AKSHAYA | Reg.No.: | 192525190 |
| Department: | AIML | Date: |  |

### ANNEXURE A

- A Booth's multiplication implementation produces an incorrect result when multiplying

-13 *9. Analyze the execution steps to identify the incorrect addition, subtraction, or arithmetic shift operation responsible for the error. Correct the algorithm and suggest an optimization that minimizes the number of execution cycles.

- A processor executing the restoring division algorithm produces an incorrect quotient for the operation 29 /5. Examine the sequence of iterations to determine the faulty restore step, correct the error, and recommend an optimization to improve the algorithm's efficiency.

- During the execution of the non-restoring division algorithm, incorrect quotient bits are generated after the fourth iteration. Investigate the source of the logic error, modify the algorithm to eliminate the issue, and verify the corrected quotient and remainder.

- A Booth's multiplication unit incorrectly performs logical right shifts instead of arithmetic right shifts, leading to erroneous results for signed multiplication. Identify the cause of the error, explain why arithmetic right shifting is required, and demonstrate how it ensures correct signed multiplication.

- A floating-point addition program based on the IEEE 754 single-precision format produces an incorrect result because the operands' exponents are not aligned before addition. Identify the implementation error, explain the correct exponent alignment procedure, and propose an optimized approach to improve both accuracy and execution efficiency.

Code of Conduct:

I  P.AKSHAYA(192525190)	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

### MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

Q1. Booth's Multiplication for (-13) × 9

Given:

M=−13,Q=9M = -13,\quad Q = 9M=−13,Q=9

Using 5-bit 2's complement:

M=−13=10011M = -13 = 10011M=−13=10011 −Q=+13=01101-Q = +13 = 01101−Q=+13=01101 Q=9=01001Q = 9 = 01001Q=9=01001

Initialize:

A = 00000

Q = 01001

Q-1 = 0

M = 10011

Booth's Multiplication Steps

| Step | A | Q | Q-1 | Operation |
| --- | --- | --- | --- | --- |
| Initial | 00000 | 01001 | 0 | Initial Values |
| 1 | 10011 | 01001 | 0 | A = A - M |
| Shift | 11001 | 10100 | 1 | Arithmetic Shift |
| 2 | 11001 | 10100 | 1 | No Operation |
| Shift | 11100 | 11010 | 0 | Arithmetic Shift |
| 3 | 01111 | 11010 | 0 | A = A + M |
| Shift | 00111 | 11101 | 0 | Arithmetic Shift |
| 4 | 10100 | 11101 | 0 | A = A - M |
| Shift | 11010 | 01110 | 1 | Arithmetic Shift |
| 5 | 11010 | 01110 | 1 | No Operation |
| Shift | 11101 | 00111 | 0 | Arithmetic Shift |

Final Product:

AQ = 1110001011

11100010112=−117101110001011_2 = -117_{10}11100010112​=−11710​

Correct Answer

(−13)×9=−117(-13)\times 9 = -117(−13)×9=−117

Error Identification

The incorrect result occurs because of:

- Wrong addition/subtraction operation during Booth encoding or

- Incorrect arithmetic right shift (sign bit not preserved).

The most common error is performing a logical shift instead of an arithmetic shift.

Correction

- Use Booth's rules correctly:

  - 01 → A = A + M

  - 10 → A = A − M

  - 00 or 11 → No Operation

- Perform Arithmetic Right Shift after every cycle.

Optimization

Use Modified Booth's Algorithm (Radix-4).

Advantages:

- Processes 2 bits at a time.

- Reduces the number of execution cycles by nearly 50%.

- Improves multiplication speed.

Conclusion

The correct result of (-13 × 9) is -117. The error is caused by an incorrect add/subtract operation or an incorrect arithmetic shift. Using Modified Booth's Algorithm reduces execution cycles and improves performance.

Top of Form

Q2. Restoring Division Algorithm for 29 ÷ 5

Given:

Dividend = 29 = (11101)₂

Divisor = 5 = (00101)₂

Expected Result:

Quotient = 5, Remainder = 4

Solution

Initial Values:

A = 00000

Q = 11101

M = 00101

Iteration 1

Shift Left (A,Q)

A = 00001

Q = 11010

Subtract M

A = 00001 - 00101

A = 11100 (Negative)

Restore A

A = 11100 + 00101

A = 00001

Set Q₀ = 0

Iteration 2

Shift Left

A = 00011

Q = 10100

Subtract M

A = 00011 - 00101

A = 11110 (Negative)

Restore A

A = 11110 + 00101

A = 00011

Set Q₀ = 0

Iteration 3

Shift Left

A = 00111

Q = 01000

Subtract M

A = 00111 - 00101

A = 00010 (Positive)

Set Q₀ = 1

Q = 01001

Iteration 4

Shift Left

A = 00100

Q = 10010

Subtract M

A = 00100 - 00101

A = 11111 (Negative)

Restore A

A = 11111 + 00101

A = 00100

Set Q₀ = 0

Iteration 5

Shift Left

A = 01001

Q = 00100

Subtract M

A = 01001 - 00101

A = 00100 (Positive)

Set Q₀ = 1

Q = 00101

Final Result

Quotient  = 00101₂ = 5

Remainder = 00100₂ = 4

Therefore,

29 ÷ 5 = 5

Remainder = 4

Faulty Restore Step

The error occurs when a negative value of A is obtained after subtraction and the restore operation is not performed correctly.

Example:

A = 11100 (Negative)

Correct Restore:

A = 11100 + 00101

A = 00001

If this restore step is skipped, the quotient becomes incorrect.

Correction

Whenever A becomes negative after subtraction, restore it by adding the divisor back:

A = A + M

and set

Q₀ = 0

Optimization

Use the Non-Restoring Division Algorithm.

Advantages:

- Eliminates the restore operation.

- Requires fewer arithmetic operations.

- Reduces execution time.

- Improves processor efficiency.

Conclusion

The correct result of 29 ÷ 5 is Quotient = 5 and Remainder = 4. The incorrect quotient is caused by a faulty restore step after a negative subtraction. The error can be corrected by properly restoring the accumulator. Using the Non-Restoring Division Algorithm further improves efficiency by reducing the number of operations and execution cycles.

Bottom of Form

Q3. Non-Restoring Division Algorithm

During the execution of the non-restoring division algorithm, incorrect quotient bits were generated after the 4th iteration.

Source of Error

In the non-restoring division algorithm:

- If A is positive, perform:

A = A - M

- If A is negative, perform:

A = A + M

The error occurred after the 4th iteration because the algorithm performed the wrong operation on A (subtraction instead of addition, or vice versa).

As a result, the quotient bit was set incorrectly.

Faulty Step

Suppose after the 4th iteration:

A = Negative

Correct operation:

A = A + M

Incorrect operation performed:

A = A - M

This produced wrong quotient bits in the remaining iterations.

Corrected Algorithm

- Shift (A,Q) left.

- If A ≥ 0, perform:

A = A - M

- If A < 0, perform:

A = A + M

- If A ≥ 0, set:

Q₀ = 1

- If A < 0, set:

Q₀ = 0

- Repeat for all bits.

- If A is negative after the final iteration:

A = A + M

Verification

Assume:

Dividend = 29

Divisor = 5

Correct Result:

Quotient  = 00101₂ = 5

Remainder = 00100₂ = 4

Check:

Dividend = Divisor × Quotient + Remainder

= 5 × 5 + 4

= 29

Hence, the result is verified.

Optimization

Use an improved non-restoring divider with:

- Faster add/subtract hardware

- Parallel shift operations

- Pipeline execution

Advantages:

- Fewer clock cycles

- Faster quotient generation

- Better processor performance

Conclusion

The incorrect quotient bits were caused by applying the wrong add/subtract operation after the 4th iteration when the sign of the accumulator was not checked correctly. After correcting the decision logic, the algorithm produces the correct result:

Quotient = 5

Remainder = 4

The efficiency of the algorithm can be improved using optimized non-restoring division hardware and pipelined execution.

4. Booth's Multiplication Using Logical Right Shift Instead of Arithmetic Right Shift

Cause of Error

The error occurs because the Booth's multiplication unit performs a logical right shift instead of an arithmetic right shift.

In a logical right shift, the leftmost bit is always filled with 0.

10110  → 01011

This changes the sign of a negative number and produces an incorrect result.

Why Arithmetic Right Shift is Required

Booth's algorithm is used for signed multiplication and works with numbers in 2's complement form.

To preserve the sign of the number during shifting, an arithmetic right shift must be used.

In an arithmetic right shift, the sign bit is copied into the new leftmost position.

Example:

10110  → 11011

Here, the sign bit 1 is preserved, so the number remains negative.

Demonstration

Consider the negative number:

-10 = 10110₂

Logical Right Shift

10110 → 01011

Result becomes positive and incorrect.

Arithmetic Right Shift

10110 → 11011

Result remains negative and correct.

Correct Operation in Booth's Algorithm

After every addition or subtraction step:

(A, Q, Q-1)

must undergo an arithmetic right shift.

This preserves the sign bit and maintains the correct partial product throughout the multiplication process.

Conclusion

The incorrect result is caused by using a logical right shift, which changes the sign of negative numbers. Booth's algorithm requires an arithmetic right shift because it preserves the sign bit in 2's complement representation. Therefore, arithmetic right shifting ensures correct signed multiplication and produces the correct final product.

5. IEEE 754 Floating-Point Addition

Implementation Error

The error occurs because the exponents of the two floating-point numbers are not aligned before addition.

As a result, the mantissas are added directly even though they represent values with different powers of 2, producing an incorrect result.

Correct Exponent Alignment Procedure

Before adding two floating-point numbers:

- Compare the exponents.

- Identify the smaller exponent.

- Right shift the mantissa of the number with the smaller exponent until both exponents become equal.

- Add the mantissas.

- Normalize the result.

- Round the result if necessary.

- Store the final sign, exponent, and mantissa.

Example:

Number 1 : 1.101 × 2⁵

Number 2 : 1.010 × 2³

Align exponents:

1.010 × 2³

= 0.01010 × 2⁵

Now add:

1.10100

0.01010

--------

1.11110

The addition is now correct because both numbers have the same exponent.

Correction

The program should first perform exponent comparison and alignment before mantissa addition.

Without exponent alignment:

Mantissa₁ + Mantissa₂  ✗

Correct method:

Align Exponents → Add Mantissas → Normalize Result ✓

Optimized Approach

To improve accuracy and execution efficiency:

- Use a dedicated Floating Point Unit (FPU).

- Perform exponent comparison and shifting in hardware.

- Use guard, round, and sticky bits for accurate rounding.

- Normalize the result automatically after addition.

Advantages:

- Faster execution.

- Higher accuracy.

- Reduced rounding errors.

- Efficient IEEE 754 compliance.

Conclusion

The incorrect result is caused by adding mantissas without first aligning the exponents. The correct IEEE 754 procedure is to align the exponents, add the mantissas, normalize the result, and then round it if required. Using hardware-based exponent alignment and normalization improves both the accuracy and efficiency of floating-point addition.